| Sr.No | Safety Zone Creation Check point | Remarks: Evidence Mandatory |
| --- | --- | --- |
|  | Before Isolation |  |
| 1 | Has all mandatory PPEs/Job specific PPEs given to all engaged manpower? Team have used it. | Evidence must be attached. (Worker wearing PPEs). |
| 2 | Is Electrical (11kV rated) hand gloves using by line man & Technician? | Evidence must be attached electrical Hand gloves is wearing by technician.. |
| 3 | Whether tools and ladder available at site is in good condition? | Evidence must be attached of tools & ladder. |
| 4 | Has isolation point as per required circuit area identified? (One pole ahead the circuit area) | Evidence must be attached of isolation points. |
| 5 | Is TBT Conducted before starting the work? | Evidence must be attached (TBT Photo.) |
|  | After Isolation |  |
| 1 | Has job specific HIRA explained to all workman and supervisor? | Evidence must be selected from drop down of TBT’s HIRA. |
| 2 | Whether safe working zone is created by using Discharge rod/ shorting chain (on both sides of HT & LT Side of DT) at the site? | Evidence must be attached of discharge rod & shorting chain fixed in DT Meter both side at site |
| 3 | Ladder shall be used for access for height work. | Evidence must be attached of ladder. |
| 4 | Essentials PPEs available & Technician have used safety belt, electrical hand gloves, safety shoes, reflective jacket & safety helmet | Evidence must be attached of full body harness. |
|  | Before Removal of Isolation |  |
| 1 | Has all workman/ job supervisor/Engineer removed from isolated circuit area? | Evidence must be attached of the location. |

Before You Start Annotating (Setup)

Step 1: Create label list (ONCE)

Create these labels exactly as written:

helmet

reflective_jacket

safety_shoes

electrical_gloves

ladder

tools

isolation_device

discharge_rod

shorting_chain

full_body_harness

person

⚠️ Do NOT add:

no_helmet

helmet_missing

not_wearing_gloves

Always start with person

If a human is visible:

- Draw a bounding box around the full body

- Head to foot (even if partially visible)

📌 Why?

- Needed for empty-zone logic

- Helps PPE context

🪖 helmet — How to annotate

Annotate helmet ONLY if:

- Helmet is visible on head

✅ Draw box tightly around helmet

❌ Do NOT annotate if:

- Helmet is in hand

- Helmet is on ground

- Helmet is hanging somewhere

- Person has no helmet

📌 In those cases → no helmet box

reflective_jacket — How to annotate

Annotate ONLY if:

- Reflective jacket is worn

✅ Box around jacket area (upper body)

❌ Do NOT annotate if:

- Normal shirt

- Jacket open but not reflective

- Jacket tied to waist

safety_shoes — How to annotate

Annotate ONLY if:

- Safety shoes clearly visible

✅ Box shoe area (one box per shoe OR one combined box)

❌ Do NOT annotate:

- Slippers

- Normal shoes

- Bare feet

electrical_gloves — How to annotate (CRITICAL)

Annotate ONLY if:

- Electrical/insulated gloves are worn

✅ Tight box around glove on hand

❌ Do NOT annotate:

- Cloth gloves

- Bare hands

- Normal rubber gloves

📌 This class must be very strict

🪢 full_body_harness — How to annotate

Annotate ONLY if:

- Full body harness is worn (straps visible)

✅ Box covering harness structure

❌ Do NOT annotate:

- Waist belt only

- Rope

- Partial harness

🪜 ladder — How to annotate

Annotate ONLY if:

- Ladder is visible (wood / metal)

✅ Box entire ladder

❌ Do NOT annotate:

- Pole steps

- Improvised climbing

🔧 tools — How to annotate

Annotate ONLY if:

- Recognizable tools visible

Examples:

- Spanner

- Pliers

- Tool kit

- Screwdriver

✅ Box tools or tool kit

❌ Do NOT annotate:

- Random metal objects

- Machinery parts

⚡ isolation_device — How to annotate

Annotate ONLY if:

- Fuse cut

- Isolator switch

- Lockout device

✅ Box ONLY the isolator device (not whole pole)

❌ Do NOT annotate:

- Normal pole

- Wires only

⚡ discharge_rod — How to annotate

Annotate ONLY if:

- Discharge rod is attached or being used

✅ Box entire rod

❌ Do NOT annotate:

- Rod lying unused

- Stick / pipe

🔗 shorting_chain — How to annotate

Annotate ONLY if:

- Chain connected to HT/LT side

✅ Box chain clearly

❌ Do NOT annotate:

- Loose chain on ground

- Other chains

Example Scenarios (VERY IMPORTANT)

Scenario 1: Worker wearing helmet + jacket, no gloves

Annotate:

person

helmet

reflective_jacket

Do NOT annotate:

electrical_gloves

Scenario 2: Worker without helmet, wearing gloves

Annotate:

person

electrical_gloves

Do NOT annotate:

helmet

Scenario 3: Empty work zone

Annotate:

(nothing)

No person → empty zone = compliant

5️⃣ Annotation Quality Rules (STRICT)

Follow these rules:

- Boxes should be tight, not loose

- Do not overlap boxes unnecessarily

- One object = one box

- Do not guess objects

- If unsure → don’t annotate

WHAT PHOTOS YOU NEED — CLASS BY CLASS (TABLE FORMAT)

| Class | Photos you MUST collect — Correct use (✅) | Photos you MUST collect — Incorrect / Missing (❌) | Notes for field team |
| --- | --- | --- | --- |
| helmet | - Front view of worker - Side profile - Worker climbing ladder/pole - Group of workers - Different helmet colors | - Bare head - Helmet in hand - Helmet hanging on pole - Helmet lying on ground - Helmet loosely placed | Model must learn presence + absence |
| reflective_jacket | - Full upper body visible - Jacket partly covered by harness - Jacket unzipped but worn - Different colors | - Normal shirt only - Jacket tied around waist - Jacket kept aside | Capture different lighting + angles |
| safety_shoes | - Standing worker (clear feet) - Worker climbing ladder - Muddy/dusty shoes - Different shoe styles | - Slippers - Sneakers - Bare feet - Normal office shoes | Feet must be clearly visible |
| electrical_gloves | - Close-up of hands - Worker holding tools - Gloves near electric wire - Gloves in sunlight + shadow | - Bare hands - Cloth gloves - Normal rubber gloves - Gloves on ground | Real site photos only (no online data) |
| ladder | - Ladder leaning on pole - Ladder on ground - Worker climbing ladder - Metal + wooden ladders | - Worker climbing pole directly - Empty pole (no ladder) | Take both near & far shots |
| tools | - Tools on ground - Tools in worker’s hand - Open toolbox - Spanner, pliers, cutters | - Empty ground - Random metal objects | Only real tools, not machinery parts |
| isolation_device | - Fuse cut-out - Isolator switch - Lockout device on pole - Danger tag attached | - Normal pole without isolator - Only wires visible | Take close + far shots |
| discharge_rod | - Rod touching conductor - Rod attached to line - Grounding in progress | - Rod lying unused - No rod in frame | Must be clearly connected |
| shorting_chain | - Chain on HT side - Chain on LT side - Grounding chain visible | - Chain lying on ground - No chain in frame | Capture both sides of DT |
| full_body_harness | - Front view - Back view - Worker climbing pole - Harness clearly strapped | - Only waist belt - Rope instead of harness - Loose harness - No harness | Must see straps clearly |
| person | - Single worker - Multiple workers - Worker far away - Worker partially visible | - Empty pole area - No workers visible | Needed for empty-zone logic |

Final classes:

helmet  , reflective_jacket  ,safety_shoes  ,electrical_gloves  , ladder  , tools, isolation_device  , discharge_rod  , shorting_chain  , full_body_harness , person

Do NOT create classes like:

no_helmet, helmet_missing, no_gloves, wrong_harness, etc.

HOW TO ANNOTATE — IMAGE BY IMAGE (WITH CASES)

For every image, follow this exact sequence:

✅ Step 1 — Always look for people first

If you see a person → draw a box around the full body.

Case A — One worker present

You draw:

person

Case B — Three workers present

You draw:

person (box 1)

person (box 2)

person (box 3)

Case C — Empty work zone

You draw:

nothing

(No person = empty zone)

✅ Step 2 — Annotate PPE only if VISIBLE

Case 1 — Worker wearing helmet + jacket + gloves, but no shoes

You draw boxes for:

person

helmet

reflective_jacket

electrical_gloves

You DO NOT draw:

safety_shoes

(Absence means non-compliance)

Case 2 — Worker with no helmet but wearing gloves

You draw:

person

electrical_gloves

You DO NOT draw:

helmet

Case 3 — Worker wearing everything correctly

You draw:

person

helmet

reflective_jacket

safety_shoes

electrical_gloves

full_body_harness (if visible)

Case 4 — Worker wearing only normal clothes

You draw:

person

Nothing else.

✅ Step 3 — Annotate equipment if present

LADDER

If ladder is visible:

ladder

If no ladder:

nothing

TOOLS

If you see clear tools:

tools

If only random metal or nothing:

do not annotate

ISOLATION DEVICE

If you see:

- Fuse cut

- Isolator switch

- Lockout device

Then draw:

isolation_device

If it’s just a normal pole:

do nothing

DISCHARGE ROD

If rod is attached to conductor:

discharge_rod

If rod is lying on ground or absent:

do nothing

SHORTING CHAIN

If chain is connected to HT/LT side:

shorting_chain

If chain is loose or absent:

do nothing

FULL BODY HARNESS

If full harness is clearly worn:

full_body_harness

If only waist belt / rope / nothing:

do nothing

3️⃣ REAL EXAMPLE IMAGES (HOW YOU WOULD LABEL THEM)

🔹 Example 1 — Ideal safe worker

Image shows:

- Worker climbing ladder

- Wearing helmet, jacket, gloves, harness

- Ladder present

You annotate:

person

helmet

reflective_jacket

electrical_gloves

full_body_harness

ladder

🔹 Example 2 — Unsafe worker

Image shows:

- Worker on pole

- No helmet, no gloves

- Only normal clothes

You annotate:

person

(That’s all — absence will be detected by your scoring logic)

🔹 Example 3 — Isolation scene

Image shows:

- Pole with isolator

- No workers

You annotate:

isolation_device

(No person box)

🔹 Example 4 — Empty zone

Image shows:

- Empty site, no workers, no tools

You annotate:

nothing

HOW TIGHT SHOULD YOUR BOXES BE?

Follow these rules:

| Object | Box rule |
| --- | --- |
| person | Full body (head to feet) |
| helmet | Tight box around helmet only |
| jacket | Upper body area only |
| shoes | Around each shoe or both together |
| gloves | Tight box around hands |
| ladder | Entire ladder |
| tools | Around tool/toolbox |
| isolation_device | Only isolator part |
| discharge_rod | Full rod |
| shorting_chain | Entire chain |
| harness | Around harness straps |